# LAB 1 REPORT — GPIO LED CONTROL (EXERCISE 3.1)

Course: Embedded Systems — IT4210E, semester 2026.1
Student: Trần Anh Dũng
Student ID: 20226031
Project: Lab1_3_1
Scope: Exercise 3.1 only

## Declaration and acknowledgement

This report describes the circuit assembled and tested during the lab session. The project is based on the sample supplied by the instructor. OpenAI Codex assisted with firmware implementation, build/flash operations, debugging, and report preparation. The hardware assembly and observation of the LED behavior were performed by the student. Sources and assistance are acknowledged below. No claim is made that all source code was independently written without assistance.

## 1. Objectives and completed work

Exercise 3.1 uses eight GPIO outputs to control eight individual LEDs. The objectives are to configure PD8–PD15 as outputs, represent the LED states using an eight-bit pattern, and implement four effects:

- One illuminated spot moving along the LED row in one direction.

- One illuminated spot moving in the opposite direction.

- All eight LEDs flashing together.

- Two symmetric illuminated spots moving from the outside toward the center and back outward.

Each effect is repeated twice before the next effect starts. The sequence runs continuously.

The class ended earlier than expected. Exercise 3.1 was completed; exercises 3.2, 3.3, and 4 were not completed and are outside the experimental results reported here. Preliminary work on exercise 3.2 is not presented as a completed assignment. This submission therefore covers only part of the full Lab 1 requirements.

## 2. Hardware and circuit

The experiment used an STM32F429 Discovery board, eight LEDs, eight series current-limiting resistors, a breadboard, jumper wires, and the board's ST-LINK USB connection. STM32CubeIDE 2.2.0 provided the compiler and build tools; STM32CubeProgrammer was used for programming and verification.

Each LED branch follows this connection:

STM32 PD pin → series resistor → LED anode (+)
LED cathode (−) → common GND → STM32 GND

Each LED has its own resistor. All branches share the board ground. An output HIGH lights its LED; an output LOW turns it off.

| LED | GPIO | Pattern bit |
| --- | --- | --- |
| LED1 | PD8 | 0 |
| LED2 | PD9 | 1 |
| LED3 | PD10 | 2 |
| LED4 | PD11 | 3 |
| LED5 | PD12 | 4 |
| LED6 | PD13 | 5 |
| LED7 | PD14 | 6 |
| LED8 | PD15 | 7 |

Physical left/right direction depends on how the board is viewed. Mode 1 traverses LED1 to LED8, and mode 2 reverses that order.

## 3. Added code and explanation

The relevant additions are in Lab1_3_1/Core/Src/main.c, inside the user-code sections. Clock initialization, HAL support, and the timer/button framework come from the supplied sample.

### 3.1 GPIO configuration

/* Section 3.1 wiring test: PD8-PD15 each drive an LED through a resistor. */
  __HAL_RCC_GPIOD_CLK_ENABLE();
  HAL_GPIO_WritePin(GPIOD, (GPIO_PIN_8 | GPIO_PIN_9 | GPIO_PIN_10 | GPIO_PIN_11 | GPIO_PIN_12 | GPIO_PIN_13 | GPIO_PIN_14 | GPIO_PIN_15), GPIO_PIN_RESET);
  GPIO_InitStruct.Pin = (GPIO_PIN_8 | GPIO_PIN_9 | GPIO_PIN_10 | GPIO_PIN_11 | GPIO_PIN_12 | GPIO_PIN_13 | GPIO_PIN_14 | GPIO_PIN_15);
  GPIO_InitStruct.Mode = GPIO_MODE_OUTPUT_PP;
  GPIO_InitStruct.Pull = GPIO_NOPULL;
  GPIO_InitStruct.Speed = GPIO_SPEED_FREQ_LOW;
  HAL_GPIO_Init(GPIOD, &GPIO_InitStruct);

__HAL_RCC_GPIOD_CLK_ENABLE() enables the clock needed to operate GPIO port D. The initial write sets PD8–PD15 LOW, so the LEDs start off. The pins are configured as push-pull outputs, without internal pull-up or pull-down resistors. Low GPIO speed is sufficient because the animation changes only every 200 ms.

### 3.2 Pattern output and four effects

/* Bit 0 controls PD8; bit 7 controls PD15. One frame lasts 200 ms. */
static void ShowLEDPattern(uint8_t pattern)
{
  uint32_t pins = ((uint32_t)pattern) << 8;
  GPIOD->BSRR = ((0xFF00U & ~pins) << 16) | pins;
  HAL_Delay(200);
}

/* Play one full cycle of the selected section 3.1 effect. */
void DisplayLEDs(int mode)
{
  static const uint8_t symmetric[] =
      {0x81, 0x42, 0x24, 0x18, 0x18, 0x24, 0x42, 0x81};

  ShowLEDPattern(0);
  switch (mode)
  {
    case 1:
      for (int bit = 0; bit < 8; bit++)
        ShowLEDPattern((uint8_t)(1U << bit));
      break;
    case 2:
      for (int bit = 7; bit >= 0; bit--)
        ShowLEDPattern((uint8_t)(1U << bit));
      break;
    case 3:
      ShowLEDPattern(0xFF);
      break;
    case 4:
      for (unsigned int step = 0; step < sizeof(symmetric); step++)
        ShowLEDPattern(symmetric[step]);
      break;
    default:
      break;
  }
  ShowLEDPattern(0);
}

#### Writing a pattern

ShowLEDPattern() receives an eight-bit pattern. A bit equal to 1 means its LED should be on; a bit equal to 0 means off. Shifting the pattern left by eight aligns bits 0–7 with port D pins 8–15.

BSRR is the GPIO bit set/reset register. Its lower half sets output pins HIGH; its upper half resets output pins LOW. The expression combines the two operations into one register write:

- pins sets the LEDs selected by the pattern.

- 0xFF00U & ~pins selects the remaining LED pins to reset.

- Shifting the reset mask left by 16 places it in the reset half of BSRR.

The mask restricts the operation to PD8–PD15, leaving the other port D outputs unchanged. HAL_Delay(200) keeps each state visible for 200 ms.

#### Modes 1 and 2: moving spot

1U << bit creates a pattern with exactly one bit set. Mode 1 increases the bit index from 0 to 7, producing 01, 02, 04, 08, 10, 20, 40, 80 in hexadecimal. Mode 2 decreases the index from 7 to 0. Both effects begin and end with all LEDs off.

#### Mode 3: flash

0xFF sets all eight bits, so all LEDs turn on. The final ShowLEDPattern(0) switches them off. Repeating this mode twice produces two flashes.

#### Mode 4: symmetric motion

The pattern array controls symmetric pairs:

| Hex pattern | Illuminated LEDs |
| --- | --- |
| 81 | LED1 and LED8 |
| 42 | LED2 and LED7 |
| 24 | LED3 and LED6 |
| 18 | LED4 and LED5 |

The array then runs in reverse order. The center pattern is repeated, keeping the center pair illuminated for 400 ms before the outward movement. This follows the repeated center state shown in the lab guide.

### 3.3 Repetition and sequencing

The following code runs inside while (1):

for (int mode = 1; mode <= 4; mode++)
    {
      for (int cycle = 0; cycle < 2; cycle++)
        DisplayLEDs(mode);
      HAL_Delay(600);
    }

The outer for loop selects modes 1 through 4. The inner loop calls the selected mode twice. A further 600 ms delay separates effects. The surrounding infinite loop restarts the complete sequence.

Modes 1, 2, and 4 each take approximately 2 seconds per cycle: eight active states plus two blank states, each lasting 200 ms. Mode 3 takes approximately 0.6 seconds per cycle. Including the four transition delays, a complete sequence takes approximately 15.6 seconds, excluding small execution overhead.

The inherited TIM6 initialization remains in the project, but these LED animations use HAL_Delay(), which relies on the HAL time base. They do not use the TIM6 counter to schedule animation frames. The implementation is blocking, which is sufficient for this standalone LED exercise but would need revision if combined with a rapidly refreshed display.

## 4. Testing and experimental results

Testing started with one LED on PD8 blinking every half second. The LED initially remained dark; correcting its polarity resolved the issue. The circuit was expanded to two LEDs, then eight. The shared ground connection was also checked after changing the breadboard wiring. All eight LEDs were confirmed blinking in the wiring test before the four-effect firmware was programmed.

The four-effect firmware was built, programmed into the STM32, verified by the programmer, and started. The build completed with zero errors and zero warnings during the session. The program implements two cycles of each effect and continuous repetition. The linked recording is supplied as the experimental demonstration.

Exercise 3.1 video: GPIO LED effects — Google Drive

The recording is the evidence for the visible sequence. Its contents and sharing permissions were not independently checked during report preparation. Successful compilation and flash verification alone do not establish that every visual requirement appears in the video.

## 5. Conclusion

Exercise 3.1 was completed using PD8–PD15 to control eight LEDs. The program uses bit patterns to generate moving, flashing, and symmetric effects, with two cycles per mode. The experiment also demonstrated why LED polarity, correct header selection, and a continuous ground return must be checked when an output does not light its LED.

Exercises 3.2, 3.3, and 4 remain incomplete because the lab session ended before they could be completed.

## 6. References

- Instructor's Lab1_IT4210E, version 2026, section 3.1: circuit, effect requirements, and sample framework. Local copy: Lab1_20252/Lab1_IT4210E.pdf.

- Instructor's lab announcements: instruction-general.md and instruction-lab-1.md.

- STMicroelectronics, UM1670 — Discovery kit with STM32F429ZI MCU, for board connector mapping consulted during assembly.

- Project source: Lab1_3_1/Core/Src/main.c; STMicroelectronics HAL and generated initialization code supplied with the project.

- OpenAI Codex: assistance with added code, debugging, build/flash operations, and report drafting.